Module: UVM-2

Project

Project - Source Code ( V1.1 to V1.2 )

- Control Sequence:

ctrl_wr

- Checking Desired Value with get( ):

- Console Output:

- Waveform:

- Setting Desired Value with set( ):

  - Console Output:

  - Waveform:

- Writing Desired Value to DUT with update( ):

  - Console Output:

  - Waveform:

- Purpose of each method:

  - get( ): Retrieves the desired value of the register from the UVM register model. This value reflects what the testbench expects, not necessarily the actual value in the DUT.

  - set( ): Sets the desired value field of the register in the UVM model. It does not directly affect the DUT but prepares the model for updates or checks.

  - update ( ): Writes the desired value from the UVM model to the DUT register, ensuring the DUT reflects the intended state defined by the testbench.

- Code:

class ctrl_wr extends uvm_sequence; `uvm_object_utils(ctrl_wr) reg_block regmodel; function new (string name = "ctrl_wr"); super.new(name); endfunction task body; uvm_status_e status; bit [3:0] wdata; bit [3:0] dv, mv; // Desired Value & Mirrored Values wdata = $urandom(); regmodel.cntrl_inst.write(status, wdata); // Check 'dv' and 'mv' Values dv = regmodel.cntrl_inst.get(); // Get Desired Value mv = regmodel.cntrl_inst.get_mirrored_value(); `uvm_info("WRITE", $sformatf(" Desired Value = %0d, Mirrored Value = %0d ", dv, mv), UVM_NONE) // Set the Desired value regmodel.cntrl_inst.set(15); // Set the Value to be 15 // Check again dv = regmodel.cntrl_inst.get(); // Get Desired Value mv = regmodel.cntrl_inst.get_mirrored_value(); `uvm_info("SET::15", $sformatf(" Desired Value = %0d, Mirrored Value = %0d ", dv, mv), UVM_NONE) // Update the Value to DUT regmodel.cntrl_inst.update(status); // Write the Value to DUT // Check again dv = regmodel.cntrl_inst.get(); // Get Desired Value mv = regmodel.cntrl_inst.get_mirrored_value(); `uvm_info("UPDATE()", $sformatf(" Desired Value = %0d, Mirrored Value = %0d ", dv, mv), UVM_NONE) endtask endclass

- Predict( ) method:

  - Difference between predict( ) & mirror( ):

Predict Method:

- Purpose: Updates the mirrored value of a register in the UVM register model based on observed activity on the DUT.

- Trigger: Typically called by monitors or predictors when a register operation (read/write) is detected on the bus.

- Behavior: It does not initiate any physical access; it simply reflects the updated state of the register.

- Use Case: A predictor observes a write operation to a register and uses predict to update the mirrored value accordingly.

### Mirror Method:

- Purpose: Compares or synchronizes the mirrored value in the UVM register model with the actual value in the DUT.

- Trigger: Can initiate a physical access (read) to retrieve the DUT's current value and update the mirrored value.

- Behavior: Provides the option to verify whether the mirrored value and the DUT value are consistent.

- Use Case: Verifying that the current state of the register in the DUT matches the expected mirrored value.

### Summary:

- predict: Passive, updates the UVM model without DUT access.

- mirror: Active, ensures consistency between the UVM model and the DUT.

  - Write(4’h5) and Predict(4’h3):

Output:

  - Mirror( UVM_CHECK ):

Output:

The error occurs because the predicted value in the UVM model was set to 4'h3, without reflecting the actual state of the register inside the DUT. When the mirror() method is called, it reads the current value from the DUT's register and compares it to the mirrored value in the UVM model using the UVM_CHECK option. If a mismatch is detected between the DUT value and the mirrored value, UVM_CHECK reports an error. After reporting the error, the mirrored value in the UVM model is updated to match the DUT's register value.

  - Observation:

First, a constant value of 4'h5 is written to the DUT using the write() method. Next, the predicted value in the UVM model is set to 4'h3, which updates the mirrored value field in the model. When the mirror() method is invoked with the UVM_CHECK option, it detects a mismatch between the mirrored value in the model and the actual value in the DUT. This mismatch triggers an error report. After reporting the error, the mirror() method updates the mirrored value in the UVM model to match the actual value in the DUT.

- FRONT_DOOR Access:

  - FRONTDOOR Wr/Rd using ctrl_wr and ctrl_rd Sequences:

    - Output:

    - Waveform:

    - Code:

///////////////////// WRITE Sequence /////////////////////////// class ctrl_wr extends uvm_sequence; `uvm_object_utils(ctrl_wr) reg_block regmodel; function new (string name = "ctrl_wr"); super.new(name); endfunction task body; uvm_status_e status; bit [3:0] wdata; bit [3:0] dv, mv; // Desired Value & Mirrored Values wdata = $urandom(); regmodel.cntrl_inst.write(status, wdata, UVM_FRONTDOOR); //Check 'dv' and 'mv' Values dv = regmodel.cntrl_inst.get(); // Get Desired Value mv = regmodel.cntrl_inst.get_mirrored_value(); `uvm_info("WRITE::FRONTDOOR", $sformatf(" Desired Value = %0d, Mirrored Value = %0d ", dv, mv), UVM_NONE) endtask endclass ///////////////////// READ Sequence /////////////////////////// class ctrl_rd extends uvm_sequence; `uvm_object_utils(ctrl_rd) reg_block regmodel; function new (string name = "ctrl_rd"); super.new(name); endfunction task body; uvm_status_e status; bit [3:0] rdata; bit [3:0] dv, mv; // Desired Value & Mirrored Values //////working with control regmodel.cntrl_inst.read(status, rdata, UVM_FRONTDOOR); //Check 'dv' and 'mv' Values dv = regmodel.cntrl_inst.get(); // Get Desired Value mv = regmodel.cntrl_inst.get_mirrored_value(); `uvm_info("READ::FRONTDOOR", $sformatf(" Desired Value = %0d, Mirrored Value = %0d ", dv, mv), UVM_NONE) endtask endclass

- BACKDOOR Access:

  - Backdoor Access in UVM:

Backdoor access allows direct interaction with DUT signals using the simulator's database, bypassing the bus protocol. Write operations deposit values directly onto register signals, and read operations sample their current values. While this method avoids triggering control logic connected to the bus, it enables efficient signal manipulation without interfering with design transitions.

### Benefits

- Backdoor access takes zero simulation time, as it avoids bus transactions.

- Useful for quickly verifying or initializing specific register states.

- Speeds up verification by complementing the standard frontdoor protocol checks.

  - HDL Paths:

    - Code:

  - BACKDOOR WRITE:

    - Output:

    - Code:

class ctrl_wr extends uvm_sequence; `uvm_object_utils(ctrl_wr) reg_block regmodel; function new (string name = "ctrl_wr"); super.new(name); endfunction task body; uvm_status_e status; bit [3:0] wdata; bit [3:0] dv, mv; // Desired Value & Mirrored Values wdata = $urandom(); regmodel.cntrl_inst.write(status, wdata, UVM_BACKDOOR); //Check 'dv' and 'mv' Values dv = regmodel.cntrl_inst.get(); // Get Desired Value mv = regmodel.cntrl_inst.get_mirrored_value(); `uvm_info("WRITE::BACKDOOR", $sformatf(" Desired Value = %0d, Mirrored Value = %0d ", dv, mv), UVM_NONE) endtask endclass

  - Peek/Poke Methods:

    - Output:

    - Code:

class ctrl_wr extends uvm_sequence; `uvm_object_utils(ctrl_wr) reg_block regmodel; function new (string name = "ctrl_wr"); super.new(name); endfunction task body; uvm_status_e status; bit [3:0] wdata; bit [3:0] dv, mv; // Desired Value & Mirrored Values wdata = $urandom(); regmodel.cntrl_inst.poke(status, wdata); //Check 'dv' and 'mv' Values dv = regmodel.cntrl_inst.get(); // Get Desired Value mv = regmodel.cntrl_inst.get_mirrored_value(); `uvm_info("POKE", $sformatf(" Desired Value = %0d, Mirrored Value = %0d ", dv, mv), UVM_NONE) endtask endclass ///////////////////////////////////////////////////////////////////////// //////////////////read data from control reg class ctrl_rd extends uvm_sequence; `uvm_object_utils(ctrl_rd) reg_block regmodel; function new (string name = "ctrl_rd"); super.new(name); endfunction task body; uvm_status_e status; bit [3:0] rdata; bit [3:0] dv, mv; // Desired Value & Mirrored Values //////working with control regmodel.cntrl_inst.peek(status, rdata); //Check 'dv' and 'mv' Values dv = regmodel.cntrl_inst.get(); // Get Desired Value mv = regmodel.cntrl_inst.get_mirrored_value(); `uvm_info("PEEK", $sformatf(" Desired Value = %0d, Mirrored Value = %0d ", dv, mv), UVM_NONE) endtask endclass

  - Poke & FRONTDOOR READ:

    - Output:

    - Code:

class ctrl_wr extends uvm_sequence; `uvm_object_utils(ctrl_wr) reg_block regmodel; function new (string name = "ctrl_wr"); super.new(name); endfunction task body; uvm_status_e status; bit [3:0] wdata; bit [3:0] dv, mv; // Desired Value & Mirrored Values wdata = $urandom(); regmodel.cntrl_inst.poke(status, wdata); //Check 'dv' and 'mv' Values dv = regmodel.cntrl_inst.get(); // Get Desired Value mv = regmodel.cntrl_inst.get_mirrored_value(); `uvm_info("POKE", $sformatf(" Desired Value = %0d, Mirrored Value = %0d ", dv, mv), UVM_NONE) endtask endclass ///////////////////////////////////////////////////////////////////////// //////////////////read data from control reg class ctrl_rd extends uvm_sequence; `uvm_object_utils(ctrl_rd) reg_block regmodel; function new (string name = "ctrl_rd"); super.new(name); endfunction task body; uvm_status_e status; bit [3:0] rdata; bit [3:0] dv, mv; // Desired Value & Mirrored Values //////working with control regmodel.cntrl_inst.read(status, rdata, UVM_FRONTDOOR); //Check 'dv' and 'mv' Values dv = regmodel.cntrl_inst.get(); // Get Desired Value mv = regmodel.cntrl_inst.get_mirrored_value(); `uvm_info("READ::FRONTDOOR", $sformatf(" Desired Value = %0d, Mirrored Value = %0d ", dv, mv), UVM_NONE) endtask endclass

  - FRONTDOOR Write & PEEK:

    - Output:

    - Code:

class ctrl_wr extends uvm_sequence; `uvm_object_utils(ctrl_wr) reg_block regmodel; function new (string name = "ctrl_wr"); super.new(name); endfunction task body; uvm_status_e status; bit [3:0] wdata; bit [3:0] dv, mv; // Desired Value & Mirrored Values wdata = $urandom(); regmodel.cntrl_inst.write(status, wdata, UVM_FRONTDOOR); //Check 'dv' and 'mv' Values dv = regmodel.cntrl_inst.get(); // Get Desired Value mv = regmodel.cntrl_inst.get_mirrored_value(); `uvm_info("WRITE::FRONTDOOR", $sformatf(" Desired Value = %0d, Mirrored Value = %0d ", dv, mv), UVM_NONE) endtask endclass ///////////////////////////////////////////////////////////////////////// //////////////////read data from control reg class ctrl_rd extends uvm_sequence; `uvm_object_utils(ctrl_rd) reg_block regmodel; function new (string name = "ctrl_rd"); super.new(name); endfunction task body; uvm_status_e status; bit [3:0] rdata; bit [3:0] dv, mv; // Desired Value & Mirrored Values //////working with control regmodel.cntrl_inst.peek(status, rdata); //Check 'dv' and 'mv' Values dv = regmodel.cntrl_inst.get(); // Get Desired Value mv = regmodel.cntrl_inst.get_mirrored_value(); `uvm_info("PEEK", $sformatf(" Desired Value = %0d, Mirrored Value = %0d ", dv, mv), UVM_NONE) endtask endclass